DC-D4517RX(2.8mm, 4.3mm)

Architectural and Engineering Specifications

Version 1.1

(Feb. 27, 2025)

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Level 1 - 28.20.00 – Video Surveillance

Level 2 - 28.21.00 – Surveillance Cameras

Level 3 - 28.21.13 – IP Cameras

Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 13493, Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

General

Product Description

DC-D4517RX is an indoorr dome type IP Camera followed by NDAA Compliance designed and manufactured by IDIS. This camera provides 5MP (2592 x 1944) resolution at 30ips (images per second) with H.265, H.264, and M-JPEG compression. This camera is equipped with Fixed-focal lens, IR LEDs, True Day/Night, PoE (IEEE 802.3af Class 3) and provides continuous operation in subzero temperature.

General Specification

- The IP camera shall be equipped with 5 Megapixel 1/2.8” CMOS Sensor.

- The IP camera shall be equipped with 2.8mm/4.3mm fixed-focal lens, F2.0/F1.6.

- The IP camera shall be a true day/night camera with a mechanical filter for low light performance.
The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).

- The IP camera shall have wide dynamic range compensation (True WDR) for improved video quality in high-contrast situations (120 dB).

- The IP camera shall be equipped with 4ea Infrared LED with range up to 20m (65.6ft).

- The IP camera shall support fixed-Iris.

- The IP camera shall utilize 2DNR/3DNR (Dynamic Noise Reduction) technology to reduce the bitrate and storage requirements by removing noise artifacts.

- The IP camera shall be equipped with 10/100 Base-T, auto-sensing, half/full duplex, RJ-45 Ethernet connection.

- The IP camera shall support industry standard Power over Ethernet (PoE) IEEE 802.3af, Class 3 to supply power to the camera over the network.

- Using IDIS NLTSrec(Non-Linear Time Shifting recording) technology, the IP camera can store the recording data to the internal recording memory buffer (60MB) in camera if there is a delay in data transmission due to the instantaneous load of the recorder or network, and then transmits the stored data to IDIS recorder safely.

- The IP camera shall deliver maximum video resolution of 2592x1944 at rates up to 30ips (images per second).

- The IP camera shall provide network connection using H.265, H.264 and MJPEG** compression.

- The IP camera shall support quadruple streams in DirectIP 2.0 protocol mode.

- The IP camera shall conform to the ONVIF Profile S/T.

- The IP camera shall be equipped with embedded web server (IDIS Web) which works independently using a Web Browser with ActiveX plug-in.

- The IP camera shall have Digest authentication, Password Protection, User access log, IP Filtering, IEEE802.1x(EAP-TLS), Chained Fingerprint, SSL Encryption and HTTPS Encryption for greater security.

- The IP camera shall have network bandwidth limitation and MAT features for more efficient use of network bandwidth.

- The IP camera shall have Easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.

- The IP camera shall have Intelligent Video Analysis (VA): Video Motion Detection, Active Tampering Alarm, Trip Zone

Protocol Specification: DirectIP 2.0

- The IP camera shall have DirectIP 2.0 mode.

- DirectIP 2.0 protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

- DirectIP 2.0 shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

- DirectIP 2.0 shall support camera can be linked to IDIS software solution such as IDIS Center and IDIS Solution Suite, or 3rd party solution while it is being connected to a DirectIP NVR.

- DirectIP 2.0 camera shall be compatible with DirectIP 1.0 NVR as well as DirectIP 2.0 NVR.

- DirectIP 2.0 camera shall be unavailable for No-password login when connecting to DirectIP 2.0 NVR and IDIS Software Solutions.

- DirectIP 2.0 protocol shall provide Quadruple streams.

- DirectIP 2.0 protocol shall support H.264 and H.265 and MJPEG compression.

Technical Specification

Video Specification

- Image Sensor: 1/2.8” CMOS

- Maximum Resolution: 2592 x 1944

- Scanning Mode: Progressive Scan

- Lens Type: Fixed-focal (f=2.8/4.3mm, F2.0/F1.6)

- Iris Control: fixed-Iris

- Angular Field of View (H: Horizontal, V: Vertical, D: Diagonal):

[DC-D4512WERA 2.8mm]

4:3 mode : 104.8º(H) 76.9º(V) 130º(D)

16:9 mode : 104.8º(H) 57.2º(V) 123º(D)

[DC-D4512WERA 4.3mm]

4:3 mode : 80º(H) 56.7º(V) 106.9º(D)

16:9 mode : 80º(H) 41.1º(V) 95.4º(D)

- Pan/Tilt/Rotate Range :

[DC-D4517RXP 2.8mm]

Pan: -170° ~ 170°, Tilt: -5° ~ 67.5°, Rotate: -109° ~ 109°

[DC-D4517RXP 4.3mm]

Pan: -170° ~ 170°, Tilt: -5° ~ 67.5°, Rotate: -109° ~ 109°

- Minimum Illumination:

[DC-D4517RX 2.8mm]

A. COLOR : 0.15 lux @ F2.0

B. B/W : 0 lux  (IR LED ON)

[DC-D4517RX 4.3mm]

A. COLOR : 0.15 lux @ F1.6

B. B/W : 0 lux  (IR LED ON)

- S/N Ratio: More than 45dB

- Maximum Frame Rate:

  - 4:3 mode : 30ips : 2592 x 1944(WDR)

  - 16:9 mode : 30ips : 2592 x 1456(WDR)

- Video Resolution:

A. 4:3 mode : 2592x1944, 1920x1440, 1280x960, 640x480

B. 16:9 mode : 2592x1456, 1920x1080, 1280x720, 640x360

- Video Compression : H.265, H.264, MJPEG

- Video Compression Level: 4 levels: Basic, Standard, High, Very High

- Intelligent Codec is supported.

- Multi-Video Streaming: Quadruple streams

- Dynamic Range: 120dB

- True Day & Night: Yes (ICR)

- IR Distance (The number of LEDs, IR wavelength): 20m / 65.6ft (4 ea)

- Intelligent Video Analytic: Video Motion Detection, Active Tampering Alarm, Trip Zone

Network Specification

- Port: RJ-45 10/100 Base-T 1 port

- Network Protocols: DirectIP 2.0 Protocol, IPv4, IPv6, RTP/RTSP/TCP, RTP/RTSP/HTTP/TCP,

RTP/UDP RTSP/TCP, HTTP, HTTPS, FTP, SNTP, SMTP, FEN, mDNS, uPNP, SNMPv2

- Streaming Mode: Quadruple Streaming

Security Specification

- Digest authentication, Password Protection, User access log, IP Filtering, IEEE802.1x(EAP-TLS), Chained Fingerprint, SSL Encryption, HTTPS Encryption

- Maximum User Access: 10 (Live), 1 (Recording), 1 (Search), 2 (Admin)

Alarm and Event Specification

- Trigger Events: Motion Detection, Tampering, Trip Zone

- Event Notification: Remote S/W, Email (with Image)

  - Encryption Type: SSL

Environmental Specification

- Operating Temperature: -10°C ~ +50°C (+14°F ~ +122°F)

*Starting up at above 0°C (32°F)

- Operating Humidity: 0% to 90% non-condensing

Electrical Specification

- Power Source: PoE IEEE 802.3af (Class 3)

- Power Consumption: PoE, IEEE 802.3af (Class 3), 7.5W

- Regulatory Approvals: FCC, CE, KC

Mechanical Specification

- Dimensions (Ø x H): Ø110 x 88mm(Ø4.33” x 3.46”)

- Unit Weight: 0.2kg(0.44lb)

Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.1 | TS Team | Feb. 12. 2025 | Update |
| 1.0 | TS Team | May. 30. 2023 | Initial Release |